Paired Biomass density estimates of Antarctic krill in Bransfield Strait during the 2023/24 austral summer from a new glider-based wideband echosounder

Anthony M Cossio1, Christian S Reiss2, and George R Cutter3.

1NOAA Fisheries, Ecosystem Science Division Southwest Fisheries Science Center LA Jolla, CA

2 La Jolla, CA

3 Morrille, Maine

Abstract

We conducted a paired deployment of underwater gliders to compare the biomass density estimates within an important fishery area for Antarctic krill in the Southern Ocean. We deployed two Teledyne Webb Research Slocum G3 gliders, each equipped with a suite of oceanographic sensors and newly designed Nortek Signature100 single beam wide-band compact echosounders (70 kHz to 120 kHz) in the Bransfield Strait, South Shetland Islands during the austral summer of 2023-24. Both gliders were deployed on November 28, 2023 and recovered 61 days later on January 27, 2024. Krill biomass density estimates were calculated for each glider using two methods, all echo energy at 120 kHz (assuming all acoustic energy was Antarctic krill) and using the SHAPES methodology to delineate swarms of krill, before echo integration. Densities from both methods ranged from 31.93 g m-2 (SD 75.24 g m-2 ) to 44.76 g m-2  (SD 97.44 g m-2 ).  Small differences in biomass densities between gliders were observed, and the differences ranged from about 0.5 to 6 g m-2, suggesting that future deployments with multiple gliders can be used to efficiently sample a single larger area, or multiple areas, increasing the flexibility in survey design while ensuring data from multiple instruments can be combined. We developed a new method to minimize oversampling due to the nature of glider sampling by creating a spatial grid of 0.2 degrees longitude, and 0.1degree latitude (the approximate Rossby radius of deformation (10 km) at 64 degrees south), and averaging data within each element before bootstrapping the data in the grid. Density estimates were comparable to the other two methods with a low estimate of 44.29 g m-2 95% CI [32.03, 70.17 g m-2 ] and a high of 56.13 g m-2 95% CI [43.01, 90.61 g m-2]. Together these data continue to demonstrate the efficacy of acoustically instrumented underwater gliders for the study the pelagic environment of the southern ocean, and for providing an alternative method for developing time series for use by fisheries managers.

Introduction

Acoustic surveys are a principal method for developing ecological insights into aquatic systems, and are critical approaches for estimating the biomass of commercially important fish stocks (). The use of autonomous instruments and vehicles to conduct or augment vessel-based surveys has become more practicable as technology has progressed (). For example,  uncrewed surface vehicles () have been used to replace or augment ship-based surveys for commercially important finfish in a variety of ecosystems including the west coast of the United States (), the Bering Sea (), And off the coast of Norway (), Both USV and  Underwater gliders () have also been used in the Southern Ocean to estimate Antarctic krill biomass. The successful use of these vehicles should lead to more widespread use of these technologies as costs of ship time continue to rise and additional constraints are placed on traditional approaches to conducting stock assessment surveys.

A major concern with the use of multiple vehicles (Ships, USVs, and UUVs), of various type and configuration, is the comparability between acoustic estimates derived from each. This is especially critical if the data from these instruments must be combined to provide a single estimate for a given stock or spatial management unit () or to understand the changes in the biomass over time. This concern is both a statistical and sampling concern, because the data must be properly considered in generating estimates. But it is also a management concern, as decision makers need to be confident about the estimates that are being used to allocate catch. So considerable effort has been expended over the years to compare the impact of different vessels, and instruments that could lead to significant differences in the estimates of fish biomass.

In the Antarctic, the Commission for the Conservation of Antarctic Marine Living Resources (CCAMLR) is responsible for managing fisheries including those for Antarctic krill (Euphausia superba) and requires annual estimates of krill biomass to progress their newly developed krill management strategy. The U. S. Antarctic Marine Living Resources (U. S. AMLR) program has historically provided acoustic biomass estimates of Antarctic krill to CCAMLR since 1991 (Hewitt et al. 2003: Reiss et al. 2008) using traditional acoustic trawl methodology. These surveys were conducted, often twice each austral summer until 2011, or a single survey during winter from 2012 to 2016 (Reiss. et al. 2017).

In 2018, U. S. AMLR replaced their ship based acoustic trawl surveys with echosounder equipped autonomous underwater gliders (Reiss et al. 2021). These surveys focused on a smaller region of the historical survey area where the krill fishery is concentrated. The gliders were equipped with ASL Inc. Acoustic zooplankton and fish profilers (AZFPs) with three narrowband frequencies (38 kHz, 67.5 kHz, and 125 kHz), and have been used to conduct annual glider-based surveys to continue to provide krill biomass estimates for use by the CCAMLR in their krill management strategy. Simulation exercises (Kinzey et al. 2021) have demonstrated that the sawtooth pattern of sampling by gliders can recover the time series of biomass density estimated from ship-based surveys. Better estimates of biomass density were obtained when the number of gliders sampling increased, or when the gliders sampled to a shallower depth (effectively increasing the number of samples collected in the upper 250m of the water column).

In this study, we deploy two gliders equipped with a new single beam wide-band echosounder (Nortek Signature100 compact echosounder)., with each following the same sampling plan to understand the likely variability in biomass that might be expected in a multi-glider survey.  We propose a new method to estimate kill biomass by gridding the glider data over a fixed grid to remove the effects of oversampling, and then use a bootstrap estimate to calculate the mean and estimate the variability of the survey estimates.  We than conclude

Methods

The U. S. AMLR program deployed two Teledyne Webb Research Slocum G3 gliders (AMLR03 and AMLR04) south of Smith Island, in the Bransfield Strait on November 28, 2023 by the RVIB Laurence M. Gould (Figure 1). The general survey design is to replicate, as best as possible, the cross basin transects historically surveyed during  previous ship surveys (Reiss et al. 2008), and recent autonomous surveys (Reiss et al. 2021). In addition to the cross basin transects, the survey path is also aimed to increase sampling in areas where the fishery operates (on the south side of the Bransfield Strait), and of penguin foraging areas near King George Island. Additionally, as part of other oceanographic investigations, a repeat survey was conducted near the mouth of Gerlache Strait in SW Bransfield Strait. After completing the survey, the gliders were retrieved on January 27, 2024 south of Hoeseason Island by the RVIB Laurence M. Gould. Both gliders dove to 950m depth or 35m off of the bottom.

The two gliders were equipped with a Sea-Bird Glider Payload Conductivity, Temperature and Depth (CTD) profiler, an Aanderaa oxygen optode (model 4831), a Wetlabs Inc. FLBBC Ecopuck  that measures fluorescence, colored dissolved organic matter, and optical backscatter. Additionally, in contrast to gliders used in previous deployments, the gliders were equipped with a new compact wide-band single beam echosounder in a wetbay forward of the buoyancy pump (Nortek Signature 100 compact echosounder). These instruments are analogous to the Signature100 instruments used by the U. S. AMLR Program in their mooring program (Cutter et. al. 2022). The gliders were programmed to dive at 26 degrees so that the acoustic transducers were directed normal to the sea surface while sampling. The echosounders are single beam wideband echosounders with an operational range between 70 kHz and 120 kHz, similar to the echosounders in the Nortek Signature 100 ADCP (Cutter, et al, 2022). We sampled at 1/6 Hz, transmitting one wideband pulse followed by two narrowband pulses. The wideband pulse with frequencies of 74, 82, 91, 99, and 108 kHz was first transmitted with a 1 second delay between frequencies. This is followed by a narrowband pulse transmitted at 70 kHz with a 1-ms duration followed by a 120 kHz pulse with a 1-ms duration. The echosounders have a range of more than 300m but an artifact in the 70 kHz data at 97m, impacted data beyond this range. So the analysis range was limited to 96m at both frequencies on both gliders.

Unlike ships that sample continuously during transiting between oceanographic stations, or along defined transecy, gliders collect acoustic data during discrete, oblique dives and climbs through the water column. Thus ship-based echsounderswill sample a fixed depth range relative to the water surface, with a maximum depth dependent on the frequencies of the transducers. The glider echosounders sample a continuous range below the glider, also frequency dependent, but but sample throughout the 1000m of the dives or to the bottom in shallower water.  Each  oblique dive is called a profile. The horizontal distance traveled by the glider during each dive-climb cycle and the interval between dives is dependent on dive depth, with increased horizontal sampling resolution when dives were shallower.

Acoustic instrument calibration

The echosounders were calibrated using the standard sphere technique (Foote et al. 1981; Demer et al. 2015; Reiss et al. 2021) in the technology tank at the Southwest Fisheries Science Center (SWFSC) on November 5, 2024 before deployment and on January 16, 2025 after recovery. A 25.4 mm tungsten carbide sphere is attached to three electronic downriggers on monofilament line such that the sphere is centered under the transducer at 4.5m below the transducer face when the glider is at the surface but is angled at 26 degrees. The glider is lifted into the tank and allowed to come to rest and commanded to the 26 degree dive angle and collects data at 1/6 Hz, for 20 minutes until about 200 pings are collected. The highest ten percent of the pings are then averaged to calculate the calibration coefficient. We used the in situ salinity and water temperature data in the test tank, monitored by the CTD aboard the glider to adjust the sound speed in calculations using the echosounders. Gains are the adjusted in Echoview software to apply the calibration.

Acoustic data processing

The acoustic data were processed using Echoview software (15.0) and analyzed in two ways. First, we only processed dives, as this is when the echosounder is perpendicular to the water surface as the glider samples. Basic processing of the data included removing the first ten meters of a dive to account for any bubbles adhering to the transducer face, removing the first three meters from the transducer face to account for ringdown, and identifying the bottom echo and stepping back three meters from the bottom echo. Background noise removal was applied, followed by impulse noise removal, when necessary, to clean the data (DeRobertis and Higginbottom 2007). This process was done for both the 70 and 120 kHz data. Decibel differencing was not used to delineate krill like in previous studies (Reiss et al. 2021), instead we integrated all the echo energy, without krill delineation. This provides a ceiling for the biomass density estimates because the energy can also include other scatterers that are not krill. Cleaned data were then integrated in 5m by 5m bins and exported as area backscattering coefficient (ABC, m2m-2) from 10m from the surface until the bottom of the dive. The second method, we used the Shoal Analysis and Patch Estimation System (SHAPES, Cotezee, 2000) or ‘schools’ detection algorithm for krill school identification. School detection settings were based on Guihen et al. (2104) and are found in Table 1. School detection was run on a 3x3 dilation filter for the 120 kHz and the detection regions were applied to each narrow band frequency (70 kHz and 120 kHz) using a -75 dB threshold. A mask was then placed over the data that was not classified as a school and was considered empty water. The detected schools data was also integrated in 5m by 5m bins and exported as ABC. Once the data were exported from Echoview, analysis was continued in Matlab for both the all echo energy output and school output. The code used to process the acoustics generally integrates the 5m x 5m bins to the surface of each discrete glider dive (profile) and matches it with the oceanographic data. Conversion factors to convert ABC into density (g m-2) were generated using the Stochastic distorted-wave Born approximation (SDWBA) target strength model. Krill length frequency data (see below) were used in to create the conversion factors following SG-ASAM (2010).

Calculation of biomass density

The processing of ABC to density requires a few steps. The ABC values of 5m x 5m bins for each dive profile is first integrated to the surface. The integrated ABCs are then multiplied by the conversion factor from the krill length frequencies in order to get a density (g m-2) for each glider dive profile. We calculated a simple mean density and standard deviation across all the dive profiles following Reiss et al. (2021).

A second method of calculating the mean density was investigated as well. The area of Bransfield Strait was divided into 0.2° by 0.1° grid cells (Figure 1). This grid size approximates the length scales (~10 km) of the Rossby Radius of deformation, typically thought to define the scale of the eddies that can grow within this system  Glider profiles were gridded into the grid cells and the mean ABC of those grids were calculated. The means of the grid cells were bootstrapped 1000 times to get the mean density and confidence intervals. Areal biomass was calculated using the mean density times the area of the Bransfield Strait survey.

Krill length frequency

Because there were no concomitant net surveys in the region from which to generate a length frequency distribution, we used krill obtained from diets of penguins (Reid and Brierley 2001; Miller and Trivelpiece 2007). Length frequency data of Antarctic krill were collected from breeding gentoo penguins at Llano Point, King George Island, Antarctica, during January 2024. Penguins often spill small boluses of krill on the ground while feeding chicks. These fresh boluses were collected opportunistically from throughout the colony and immediately sorted to identify intact, whole individuals. The length of all whole individuals was measured from the anterior side of the eye to the tip of the telson.

Results

General findings

Both gliders sampled the water column toa depth of about 1000m. The gliders also covered about the same number of dives in the surveyed area of Bransfield Strait, with just a couple of differences. AMLR03 completed 579 total dives that generated 321 profiles in the survey area, while AMLR04 completed 563 total dives generating 323 profiles in the survey area. AMLR03 also sampled an additional loop off King George Island that was not sampled by AMLR04. Profiles of mean ABC showed that most krill were in the upper 250m of the water column (Figure 2). This finding is consistent with previous ship () and glider studies () that have demonstrated that most krill biomass during summer is in the upper few hundred meters of the water column. Further, because most of the Bransfield Strait is shallower than 1000m, , it is clear that there were very few krill aggregations at depth.  The surface mapped biomass density of each glider showed several areas with higher biomass densities, but in general the spatial distribution of biomass density was similar between gliders. (Figure 3A and Figure 3B), and the biomass densities were very similar demonstrating that data from these multi glider deployemnts can be combined to increase sampling efficiencies in the Southern Ocean.

Comparison of the distribution of

There were small  differences in biomass density estimates between the two frequencies used and between the two gliders when using all echo energy data (Table 2) and the Shapes algorithm (Table 3). The differences between gliders and approach. In general differences in biomass density estimates varied between 0.5 and 6 g m-2. For example, AMLR03 had higher density estimates for both the 70 kHz and 120 kHz than AMLR04. The mean density or AMLR03 using the 70 kHz was 93.64 g m-2 (SD 204.04 g m-2), and the 120 kHz data estimated a density of 151.29 g m-2 (SD 337.26 g m-2). In contrast, biomass density for AMLR04 at 70 kHz and 120 kHz was 77.32 g m-2 (SD 205.97 g m-2) and 97.16 g m-2 (SD 303.75 g m-2), respectively.

Using the schools method, similar differences in biomass density were observed (Table 4), and interestingly, very large differences in the number of schools were detected. For example, at 70 kHz the schools method identified 2059 schools from AMLR03 and 1420 schools for AMLR04. This difference increased when using the 120 kHz to identify schools where AMLR03 identified 2227 and AMLR04 identified 936, less than half.

The discrepancy in the number of schools was not expected, and further investigation of the statistical properties of the distribution of krill was conducted to understand this difference. Histograms of log10 ABC showed that the patterns of biomass density distribution were similar between the two gliders (Figure 5A), as might be expected given that one glider was following the other over the whole survey and were only a week apart at the end of the survey. Empirical cumulative distribution plots also (Figure 5B) demonstrated that the shapes of the distributions were also similar, but had an offset. Given the difference in the number of schools observed, and the similarity in shapes of the empirical distributions, this suggests that there is a calibration offset between the two gliders, and that another calibration might be necessary.

Spatial gridding and bootstrapping

The current method we employ, when using gliders, to calculate the biomass density, and estimate the total biomass of the survey area, does not consider the relative over sampling that can occur as the gliders move though areas with large tidal movements or areas of considerable shear. These areas then get oversampled relative to the goal of getting a representative sample of biomass density, and attempting to keep the transects as linear as possible. Thus, we have not presented error estimates or the biomass densities in the past (Reiss et al, 2021). To address this issue, we employed a two-fold strategy to try and address this over sampling. We developed a grid (Figure 2) across the entirety of the peninsula region that is fine enough (0.2° longitude by 0.1° latitude) representing about 100 km2 per grid cell. We than allocated the glider dives to these cells and averaged all data within a grid. This reduced the number of data points of each glider by more than half from 321 (AMLR03) and 323 (AMLR04) profiles in the un-gridded data to 115 (AMLR03) and 117 (AMLR04) in the gridded data. Each grid cell with data resulted in grid cells had from 1 to 9 profiles in a cell with an average of 2.7 profiles per grid cell or AMLR04 while AMLR03 had 1 to 13 profiles per grid cell with just three grid cells having more than 10 profiles in them.

After generating this gridded data set, we accounted for any grid to grid autocorrelation by bootstrapping to 1000 estimates of the mean biomass density, and then calculating a grand mean of those 1000 estimates. We used a bootstrapping method to calculate 95% confidence limits around each of the means. Estimates of bootstrapped mean biomass density of each glider was similar to that using the full data set (Table 5). For example, using the full echo energy data set the mean biomass of krill at 120 kHz the biomass density was within 7 percent of the gridded means for both AMLR03 and AMLR04.

This approach, and other geospatial modeling approaches have been used in a number of studies, including in the antarctic.

Total biomass estimates

The various estimates of krill biomass in the Bransfield Strait are presented in Tables 3, 4, and 5. AMLR03 seemed to have the best calibration and so we focus on those estimates here. Calculated total biomass for the Bransfield Strait, ranged from 93.64 to 151.29 g m-2 at 70 kHz and 120 kHz using the full echo energy integration approach, from 83.48 to 137.53 g m-2 at 70 kHz and 120 kHz using the schools approach, and from 101.77 to 161.95 g m-2 at 70 kHz and 120 kHz using the bootstrap approach. The difference between the two approaches is about 10 g m-2 (5 to 10%) and suggests that most of the acoustic returns at these frequencies is krill like.

Discussion

We have previously demonstrated that underwater gliders are capable of producing defensible estimates of the biomass of Antarctic krill in Antarctic waters (Reiss et al. 2021). We have also shown using simulation that the sawtooth pattern of glider movement through the water column is robust, and provides similar estimates to ships (Kinzey et al. 2022). Here we have shown that the statistical distribution properties between two gliders flying the same survey pattern is also robust. Finally, we have also demonstrated the use of a new compact single beam wide-band echosounder (Nortek Signature 100 compact echosounder) derived from a similar instrument used on moorings by the U.S. AMLR Program (Cutter et al. 2022).

Krill school parameters were slightly different than those in Guihen et al. (2014). They tested a number of different parameters as well as comparing EK60 ship schools parameters and a Seaglider equipped with an ES853 single beam echosounder. They noted that glider derived data were coherent with ship-borne measurements using school analysis.

Due to the nature of the glider producing profiles instead of constant data like a ship, we did not separate day and night profiles. To be conservative with possible bubble attenuation we removed the top 10 m of the surface data. This is similar to ship data. In reality, we could just remove the top 5 m of surface data since the likelihood of bubbles accumulating on the face of the transducer is minimal. Gliders could have much less bias in sampling krill due to diel vertical migration (Demer and Hewitt 1995) since gliders can begin sampling the water column at 3 meters. Ships using 120 kHz may not have a good signal to noise ratio past 500 m (De Robertis and Higginbottom 2007) while the glider can use the 120 kHz down to 1000 m making it a valuable tool to sample possible deep euphausiids. Ships are also inherently noisy while a glider is quiet, allow for cleaner acoustic data collection since it is buoyancy driven.

There has been  considerable effort expended over the last 20 or so years to understand the impact of varying survey ships

In general, there often differences in biomass estiomates, and in the perceived imapct of the ship noise an doperation. Many studies have tried to understand whether fdifferneces in fish bevior is a result of the noise pressure or noise field and the response of fish to these factors. Some have proposed that the effect could be a function of the perception by fish that the noises are derived from predators. And that the repsonse by

A  few studies have examined the effect of gliders on the  response of fish and zooplankton.. Guihan ad proopsed that the there were significant avoidance reactions from diving gliiders. between ships depending on all manner of

Conclusions

This is the fifth deployment of echosounder equipped gliders during austral summer around the Antarctic Peninsula, and further demonstrates that these tools can provide high quality data and can be used to  continue acoustic time series that were started in the 1990s.

In addition to providing estimates of biomass density that can be directly used for in the revised krill management plan, the data collected by these instruments have other important implications. For example, we have shown that there are few krill schools below 250m depth, and in general, because the gliders dive to 1000m they reach most of the bottom in the Bransfield strait except in the deep basins away from the fishing areas. Thus, CCAMLR can be more confident that the biomass estimates based on the integration of the upper 250m is sampling a very large fraction of the krill in the water column. We continue to explore different approaches to the calculation of biomass density

and have proposed a modification of the geospatial structuring of sampling, and demonstrated that with even fewer samples in the region (gridded into appropriate cells), the biomass density is still within about 10 to 15% of the estimates of the estimates calculated using the full echo energy data.

In the future, deploying acoustically equipped gliders later in the season may be useful in understanding the changes in vertical distribution in areas away from fishing grounds, as gliders have a long loitering time (months), and can collect data before fishing starts, and until fishing in a region ends. We are working to telemeter acoustic data back to shore during deployments, and this future development will provide us the opportunity conduct adaptive sampling in ecologically important areas.

References

Coetzee J (2000). Use of a shoal analysis and patch estimation system (SHAPES) to characterise sardine schools. Aquatic Living Resources, 13(1), 1-10.

Cutter Jr, GR, Reiss, CS, Nylund, S, and Watters, GM (2022). Antarctic krill biomass and flux measured using wideband echosounders and acoustic Doppler current profilers on submerged moorings. Frontiers in Marine Science, 9, 784469.

De Robertis, A, and Higginbottom, I. (2007). A post-processing technique to estimate the signal-to- noise ratio and remove echosounder background noise. ICES Journal of Marine Science, 64(6), 1282- 1291.

Demer, DA, Berger, L, Bernasconi, M, Bethke, E, Boswell, K, Chu, D,and Williamson, N (2015). Calibration of acoustic instruments. ICES Cooperative Research Reports (CRR), 215.

Demer, D. A., & Hewitt, R. P. (1995). Bias in acoustic biomass estimates of Euphausia superba due to diel vertical migration. Deep Sea Research Part I: Oceanographic Research Papers, 42(4), 455-475.

Foote K.G., Knudsen H.P., Vestnes G., Brede R. and Nielsen R.L. (1981). Improved calibration of hydroacoustic equipment with copper spheres. ICES CM1981/B:20, 18 pp

Guihen, D, Fielding, S. Murphy, E J, Heywood, KJ, and Griffiths, G. (2014). An assessment of the use of ocean gliders to undertake acoustic measurements of zooplankton: the distribution and density of Antarctic krill (Euphausia superba) in the Weddell Sea. Limnology and Oceanography:

Methods, 12(6), 373-389. https://doi.org/10.4319/lom.2014.12.373

Hewitt, RP, Demer DA, and Emerey, JH. (2003). An 8-year cycle in krill biomass density inferred from acoustics surveys conducted in the vicinity of South Shetland Islands during the austral summers of 1991-1992 through 2001-2002. Aquatic Living Resources. Vol 16, Issue 3. Pp. 205-213. https://doi.org/10.1016/S0990-7440(03)00019-6

Kinzey D, Cossio AM, Reiss CS and Watters GM (2022). Acoustic sampling of Antarctic krill with simulated underwater buoyancy gliders: Does the sawtooth dive pattern work? Front. Mar.

Sci. 9:1064181. doi: 10.3389/fmars.2022.1064181

Miller, AK, and Trivelpiece, WZ. (2007). Cycles of Euphausia superba recruitment evident in the diet of Pygoscelid penguins and net trawls in the South Shetland Islands, Antarctica. Polar Biology, 30, 1615-1623.

Reid, K, and Brierley, A S. (2001). The use of predator-derived krill length-frequency distributions to calculate krill target strength. CCAMLR Science, 8, 155-163.

Reiss, CS, Cossio, AM, Loeb, V, and Demer, DA. (2008). Variations in the biomass of Antarctic krill (Euphausia superba) around the South Shetland Islands, 1996–2006. ICES Journal of Marine Science, 65(4), 497-508.

Reiss CS, Cossio A, Santora JA, Dietrich KS and others (2017). Overwinter habitat selection by Antarctic krill under varying sea-ice conditions: implications for top predators and fishery management. Mar Ecol Prog Ser 568:1-16. https://doi.org/10.3354/meps12099

Reiss CS, Cossio AM, Walsh J, Cutter GR and Watters GM (2021). Glider-Based Estimates of Meso-Zooplankton Biomass Density: A Fisheries Case Study on Antarctic Krill (Euphausia superba) Around the Northern Antarctic Peninsula. Front. Mar. Sci. 8:604043. doi: 10.3389/fmars.2021.604043

SC-CAMLR (2010). Report of the fifth meeting of the Subgroup on Acoustic Survey and Analysis Methods. SC-CCAMLR-XXIX/6.

Tesler, W. D. (1989) Bias and precision in acoustic biomass estimation. Proceedings of the Institute of Acoustics, 11(3), 202-211.

WG-ASAM (2024) Report of the Working Group on Acoustic Survey and Analysis Methods (WG- ASAM-2024).

Tables

Table 1. School detection settings used to estimate the density of krill schools during the 2023/24 austral summer glider deployments using a -75 dB threshold.

| Minimum total school height (meters) | 2 |
| --- | --- |
| Minimum candidate length (meters) | 1.5 |
| Minimum candidate height (meters) | 2 |
| Maximum vertical linking distance (meters) | 2 |
| Maximum horizontal linking distance (meters) | 3 |
| Minimum total school length (meters) | 7.5 |

Table 2. Antarctic krill density and biomass estimates, standard deviations and the number of profiles in the survey analysis.

| Glider | 70 kHz density (g/m2) | 70 kHz STD | 70 kHz biomass (tons) | 120 kHz density (g/m2) | 120 kHz STD | 120 kHz biomass (tons) | Survey profiles (N) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AMLR03 | 44.76 | 97.44 | 4.0282e+05 | 35.66 | 79.43 | 3.2092e+05 | 321 |
| AMLR04 | 44.15 | 117.42 | 3.9731e+05 | 41.84 | 130.77 | 3.7655e+05 | 323 |

Table 3. Antarctic krill density estimates and standard deviation of school analysis in the survey area.

| Glider | 70 kHz density (g/m2) | 70 kHz STD |  | 120 kHz density (g/m2) | 120 kHz STD | 120 kHz school count |
| --- | --- | --- | --- | --- | --- | --- |
| AMLR03 | 39.04 | 92.16 |  | 31.93 | 75.24 | 710 |
| AMLR04 | 37.54 | 103.20 |  | 34.95 | 99.71 | 582 |

Table 4. Table of bootstrapped densities using the scvhools analysis with lower and upper confidence intervals and the number of binned data samples for the entire area using 0.2° by 0.1° grid cells.

| Glider | 70 kHz density | Lower CI | Upper CI | 120 kHz density | Lower CI | Upper CI | Number of samples |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AMLR03 | 45.15 | 36.13 | 57.24 | 34.74 | 27.04 | 45.02 | 106 |
| AMLR04 | 38.92 | 30.11 | 52.67 | 36.02 | 26.63 | 51.36 | 101 |

Figure 1. Map of the path of AMLR03 and AMLR04 Slocum gliders in the Bransfield Strait.

Figure 2. Spatial grid (0.2° by 0.1° grid cells) (red dots are the corners of the grid cells) with AMLR03 dive profile locations (blue *) overlayed.

Figure 3. Depth distribution of averaged ABC for AMLR03 (blue) and AMLR04 (orange) using the 70 kHz (A) and 120 kHz (B).

Figure 4. Maps of spatial distribution of krill densities of the 120 kHz for AMLR03 (A) and AMLR04 (B) using the schools analysis.

Figure 5. Empirical comparisons of acoustic estimates of biomass density from two gliders during the 2023/24 austral summer (A) Histograms of 120kHz log10 ABC for each profile; (B) Empirical cumulative distribution of biomass for both AMLR03 (blue) and AMLR04 (orange) using the schools analysis.

Appendix 1. Table 2. Krill length frequencies from Gentoo penguins on King George Island.

| Length (mm) | Frequency |  | Length (mm) | Frequency |
| --- | --- | --- | --- | --- |
| 20 | 0.000 |  | 41 | 0.073 |
| 21 | 0.000 |  | 42 | 0.063 |
| 22 | 0.002 |  | 43 | 0.037 |
| 23 | 0.000 |  | 44 | 0.044 |
| 24 | 0.000 |  | 45 | 0.077 |
| 25 | 0.000 |  | 46 | 0.042 |
| 26 | 0.000 |  | 47 | 0.066 |
| 27 | 0.005 |  | 48 | 0.044 |
| 28 | 0.002 |  | 49 | 0.028 |
| 29 | 0.007 |  | 50 | 0.044 |
| 30 | 0.009 |  | 51 | 0.028 |
| 31 | 0.009 |  | 52 | 0.028 |
| 32 | 0.005 |  | 53 | 0.005 |
| 33 | 0.023 |  | 54 | 0.005 |
| 34 | 0.035 |  | 55 | 0.009 |
| 35 | 0.063 |  | 56 | 0.005 |
| 36 | 0.044 |  | 57 | 0.002 |
| 37 | 0.033 |  | 58 | 0.000 |
| 38 | 0.049 |  | 59 | 0.000 |
| 39 | 0.047 |  | 60 | 0.000 |
| 40 | 0.063 |  |  |  |