# Revised biomass density estimates of Antarctic krill in Bransfield Strait during the 2023/24 austral summer from a new glider-based wideband echosounder and proposed over winter mooring deployments for austral 2025/26

Anthony M Cossio and Christian S. Reiss  (Retired)

NOAA Fisheries

Ecosystem Science Division Southwest Fisheries Science Center LA Jolla, CA

# Abstract

The U. S. Antarctic Marine Living Resources (U. S. AMLR) program deployed two Teledyne Webb Research Slocum G3 gliders, each equipped with a suite of oceanographic sensors and newly designed Nortek Signature100 single beam wide-band compact echosounder (70 kHz to 120 kHz). We used these gliders to estimate biomass densities (g m-2) of Antarctic krill (Euphausia superba) in the Bransfield Strait during the austral summer of 2023-24. Both gliders were deployed on November 28, 2023 and recovered 61 days later on January 27, 2024. Krill biomass density estimates were calculated from both gliders using two methods, using all echo energy and using schools to delineate krill. Krill length frequencies from the diets of Gentoo penguins at King George Island were used to construct conversion factors. Densities from both methods ranged from 33.91 g m-2 (SD 78.77 g m-2) to 48.37 g m-2 (106.77 g m-2), corrected from EMM2024/53. The corrected numbers were calculated from new calibration data. We developed a new method to minimize oversampling due to the nature of gliders by creating a grid and bootstrapping the data in the grid.

Density estimates were comparable to the other two methods with a low of 41.77 g m-2 95% CI [30.46,67.42 g m-2] and a high of 59.16 g m-2 95% CI [43.52, 89.97 g m-2]. Gliders

remain an important tool to continue to provide acoustic biomass estimates of Antarctic krill.

# Introduction

CCAMLR requires estimates of krill biomass at appropriate time and space scales in order to develop a revised krill management plan that can allocate krill catch within and among subareas. Annual biomass estimates have historically been made from research vessel based acoustic trawl surveys. Owing to the cost of these surveys, many national programs have reduced the frequency or eliminated surveys. At the same time, krill catch has increased, and in some areas annually reach interim catch limits. Thus, there is a realization that a revised krill management strategy will need to expand the types of surveys that can be included to generate estimates of krill biomass. Considerable effort has been expanded to use fishing vessels to provide acoustic estimates on fishing grounds, in addition to their use in fishery independent surveys.

The US Antarctic Marine Living Resources (US AMLR) program has historically provided Antarctic krill acoustic biomass estimates since 1991 (Hewitt et al. 2003: Reiss et al. 2008). Initially, the US AMLR program used ships to collect acoustic data during the austral summer until 2012, when the program did five consecutive winter acoustic estimates (2012 – 2016) (Reiss. et al. 2017). In 2018, US AMLR started using autonomous underwater gliders equipped with active acoustics (Reiss et al. 2021) and have been performing annual glider deployments to continue to provide summer krill biomass estimates.

Here we report on krill estimates or the 2023/24 austral summer in Bransfield Strait from two deployed autonomous gliders. We demonstrate the use of a new single beam wide-band echosounder (Nortek Signature100 compact echosounder), and compare the biomass density estimated by each glider to better understand the utility of these tools. Finally, we propose a new method to estimate kill biomass by gridding the glider data over a fixed grid to remove the effects of oversampling, and then use a bootstrap estimate to calculate the mean and estimate the variability of the survey estimates.

# Methods

The US AMLR program deployed two Teledyne Webb Research Slocum G3 gliders (AMLR03 and AMLR04) south of Smith Island, in the Bransfield Strait on November 28, 2023 by the RVIB Laurence M. Gould (Figure 1). The general survey design is to replicate, as best as possible, the cross basin transects historically surveyed during our previous ship surveys (Reiss et al. 2008), and recent autonomous surveys. In addition to the cross basin transects, the survey path is also aimed to increase sampling in areas where the fishery operates (on the south side of the Bransfield Strait), and of penguin foraging areas near King George Island. Additionally, as part of other oceanographic investigations, a repeat survey was conducted near the mouth of Gerlache Strait in SW Bransfield Strait. After completing the survey, the gliders were retrieved on January 27, 2024 south of Hoeseason Island by the RVIB Laurence M. Gould. Both gliders dove to 950m depth or 35m off of the bottom.

The two gliders were equipped with a Sea-Bird Glider Payload CTD, an Aanderaa oxygen optode model 4831, a Wetlabs Ecopuck (FLBBC) that measured fluorescence, colored dissolved organic matter, and optical backscatter. Additionally, in contrast to gliders used in previous deployments, the gliders were equipped with a new compact wide-band single beam echosounder in a wetbay forward of the buoyancy pump (Nortek Signature 100 compact echosounder). The gliders were programmed to dive at 26 degrees so that the acoustic transducers were directed normal to the sea surface while sampling. The echosounders are single beam wideband echosounders with an operational range between 70 kHz and 120 kHz, similar to the echosounders in the Nortek Signature 100 ADCP (Cutter, et al, 2022). We sampled at 1/6 Hz, transmitting one wideband pulse followed by two narrowband pulses. The wideband pulse with frequencies of 74, 82, 91, 99, and 108 kHz was first transmitted with a 1 second delay between frequencies. This is followed by a narrowband pulse transmitted at 70 kHz with a 1-ms duration followed by a 120 kHz pulse with a 1-ms duration. The echosounders have a range of 200m but there was an artifact in the 70 kHz data at 97m so the analysis range was limited to 96m at both frequencies on both gliders.

Unlike ships, gliders collect acoustic data during discrete, oblique dives through the water column. Ships will sample along transects and over a fixed depth range relative to the water surface. The glider acoustics sample similar to an oblique net tow, were the range of the echosounder (effective 96m in this survey) is an angled slice through the water column. Each of these discrete, oblique dives are called profiles. The horizontal distance traveled by the glider during each dive-climb cycle and the interval between dives was dependent on dive depth, with increased horizontal sampling resolution when dives were shallower, thus more frequent.

Acoustic data processing Calibration

The echosounders were originally calibrated using the standard sphere technique (Foote et al. 1981; Demer et al. 2015; Reiss et al. 2021) in the technology tank at the Southwest Fisheries Science Center (SWFSC) on March 29, 2024. After realizing the first calibration was bad, we performed more calibrations. Later calibrations took place November 5, 2024 and January 16, 2025. In March, a 25.4 mm tungsten carbide sphere was attached to the wetbay of the glider, hanging down from monofilament line on either side of the transducer. The November and January calibrations used a 25.4 mm tungsten carbide sphere attached to three electronic downriggers on monofilament line such that the sphere is centered under the transducer at 4.5m below the transducer face when the glider is at the surface but is angled at 26 degrees. The second calibration used live viewing of the data to make sure the sphere was on axis, where the first calibration didn’t use live viewing. The glider is lifted into the tank and allowed to come to rest and commanded to a 26 degree dive angle and collected data at 1/6 Hz for the March calibration and ½ Hz for the following two calibrations, for 20 minutes until about 200 pings are collected.

The highest ten percent of the pings are then averaged to calculate the calibration coefficient. We used the in situ salinity and water temperature data from the CTD aboard the glider to adjust the sound speed in calculations using the echosounders. Gains are the adjusted in Echoview software to apply the calibration.

Processing

The acoustic data were analyzed using Echoview software (15.0) and processed in two ways. First, we only processed dives, as this is when the echosounder is perpendicular to the water surface as the glider samples. Basic processing of the data included removing the first ten meters of a dive to account for any bubbles adhering to the transducer face, removing the first three meters from the transducer face to account for ringdown, and identifying the bottom echo and stepping back three meters from the bottom echo. Background noise removal was applied, followed by impulse noise removal, when necessary, to clean the data (DeRobertis and Higginbottom 2007). This process was done for both the 70 and 120 kHz data. Decibel differencing was not used to delineate krill like in previous studies (Reiss et al. 2021), instead we integrated all the echo energy, without krill delineation. This provides a ceiling for the biomass density estimates because the energy can also include other scatterers that are not krill. Cleaned data were then integrated in 5m by 5m bins and exported as area backscattering coefficient (ABC, m2m-2) from 10m from the surface until the bottom of the dive. The second method, we used the Shoal Analysis and Patch Estimation System (SHAPES, Cotezee, 2000) or ‘schools’ detection algorithm for krill school identification. School detection settings were based on Guihen et al. (2104) and are found in Table 1. School detection was run on a 3x3 dilation filter for the 120 kHz and the detection regions were applied to each narrow band frequency (70 kHz and 120 kHz) using a -75 dB threshold. A mask was then placed over the data that was not classified as a school and was considered empty water. The detected schools data was also integrated in 5m by 5m bins and exported as ABC.

Once the data were exported from Echoview, analysis was continued in Matlab for both the all echo energy output and school output. The code used to process the acoustics generally integrates the 5m x 5m bins to the surface of each discrete glider dive (profile) and matches it with the oceanographic data.

Conversion factors to convert ABC into density (g m-2) were generated using the Stochastic distorted-wave Born approximation (SDWBA) target strength model. Krill length frequency data (see below) were used in to create the conversion factors following SG-ASAM (2010).

Calculation of biomass density

The processing of ABC to density requires a few steps. The ABC values of 5m x 5m bins for each dive profile is first integrated from 250m to the surface. The integrated ABCs are then multiplied by the conversion factor from the krill length frequencies in order to get a density (g m-2) for each glider dive profile. We calculated a simple mean density and standard deviation across all the dive profiles following Reiss et al. (2021).

A second method of calculating the mean density was investigated as well. The area of Bransfield Strait was divided into 0.2° by 0.1° grid cells (Figure 1). Glider profiles were gridded into the grid cells and the mean ABC of those grids were calculated. The means of the grid cells were bootstrapped 1000 times to get the mean density and confidence intervals. Areal biomass was calculated using the mean density times the area of the Bransfield Strait survey.

Krill length frequency

Because there were no concomitant net surveys in the region from which to generate a length frequency distribution, we used krill obtained from diets of penguins (Reid and Brierley 2001; Miller and Trivelpiece 2007). Length frequency data of Antarctic krill were collected from breeding gentoo penguins at Llano Point, King George Island, Antarctica, during January 2024. Penguins often spill small boluses of krill on the ground while feeding chicks. These fresh boluses were collected opportunistically from throughout the colony and immediately sorted to identify intact, whole individuals. The length of all whole individuals was measured from the anterior side of the eye to the tip of the telson.

# Results and Discussion

General findings

Both gliders covered about the same number of dives in the surveyed area of Bransfield Strait, with just a couple of differences. AMLR03 completed 561 total dives that generated 321 profiles in the survey area, while AMLR04 completed 563 total dives generating 323 profiles in the survey area. AMLR03 also sampled an additional loop off King George Island that was not sampled by AMLR04. Profiles of mean ABC backscatter showed that most krill were in the upper 250m of the water column (Figure 2), consistent with previous studies. Importantly, because gliders can sample the whole water column, there is less concern here that we are missing a large fraction of krill at depth compared to ship surveys in this area. In fact, no significant aggregations weere observed below about 500m. The surface mapped biomass density of each glider showed several areas with higher biomass densities, but in general the spatial distribution of biomass density was similar between gliders. (Figure 3A and Figure 3B).

There were differences in biomass density estimates between the two frequencies used and between the two gliders when using all echo energy data in the survey area (Table 3). The estimated krill densities from AMLR03 survey area were 10 g m-2 between the two frequencies while AMLR04 had a difference of 2 g m-2 between the two frequencies. The mean density or AMLR03 using the 70 kHz was 48.37 g m-2 (SD 106.77 g m-2), and the 120 kHz data estimated a density of 38.22 g m-2 (SD 85.64 g m-2). In contrast, biomass density for AMLR04 at 70 kHz and 120 kHz was 43.33 g m-2 (SD 117.38 g m-2) and 41.67 g m-2 (SD 130.78 g m-2), respectively.

Using the schools method, similar differences in biomass density were observed (Table 4), and interestingly, large differences in the number of schools were detected. AMLR03 detected 635 schools while AMLR04 detected 515 in the upper 250 m. The densities between the gliders were similar to the all echo energy data. AMLR03 had a density of 41.89 g m-2 (SD 99.28 g m-2) for the 70 kHz and 33.91 g m-2 (SD 78.77 g m-2) for the 120 kHz. The 70 kHz for AMLR04 density was 37.62 g m-2 (SD 103.22 g m-2) and 35.01 g m-2 (SD 99.72 g m-2) for the 120 kHz.

The discrepancy in the number of schools was not expected, and further investigation of the statistical properties of the distribution of krill was conducted to understand this difference. Histograms of log10 ABC showed that the patterns of biomass density distribution were similar between the two gliders (Figure 4A), as might be expected given that one glider was following the other over the whole survey and were only a week apart at the end of the survey. Empirical cumulative distribution plots also (Figure 4B) demonstrated that the shapes of the distributions were also similar, but had an offset.

The calibration coefficients applied to the two echosounders were noticeably different compared to the first time calibrating the echosounders. The first incorrect calibration had a 0.5 dB difference between the two echosounders at 70 kHz and a 1.83 dB difference between the two echosounders at 120 kHz.. After the second calibration these differences were reduced to a 0.3 dB difference between the two echosounders at 70 kHz, and a 0.79 dB difference  for the 120 kHz channel of AMLR03 and AMLR04.

Each glider was recalibrated twice during the same day (AMLR03 on November 5, 2024 and AMLR04 on January 16, 2025).  The recalibrations were done with downriggers, similar to a ship, and live viewing the data to ensure that the sphere was on axis of the center beam. This method is more precise than the previous method used in EMM2024/53. (an issue we are still taking the upper 10% of the values and averaging them (do we know how many pings were  averaged).

Each of those calibrations were within one decibel of each other providing confidence that the calibration was done correctly.

Spatial gridding and bootstrapping

The current method employed when using gliders to calculate the biomass density, and estimate the total biomass of the survey area, does not consider the relative over sampling that can occur as the gliders move though areas with large tidal movements or areas of considerable shear. In these situations the glider does not make headway relative to the movements of the water itself. Thus, these areas then get oversampled relative to the goal of getting a representative sample of biomass density along prescribed transects. Thus, we have not presented error estimates for

the biomass densities in the past (Reiss et al, 2021). To address this issue, we employed a two-fold strategy minimize the effects of this oversampling. We developed a grid (Figure 1) across the entirety of the peninsula region that is fine enough (0.2° by 0.1°) to represent about horizontal distances of about 10km in longitude and latitude. . This spatial dimension is approximately the Rossby radius of deformation, which dictates the length scale of the eddies formed at these latitudes. We then allocated the glider dives to these cells and averaged all data within a grid cell. This reduced the number of data points of each glider by more than half from 321 (AMLR03) and 323 (AMLR04) profiles in the un-gridded data to 74 (AMLR03) and 72 (AMLR04) in the gridded data. Each grid cell with data resulted in grid cells had from 1 to 19 profiles in a cell with an average of 4.5 profiles per grid cell for AMLR04 while AMLR03 had from 1 to 15 profiles per grid cell with an average of 4.3 profiles per grid cell.

After generating this gridded data set, we accounted for any spatial autocorrelation by bootstrapping  1000 estimates of the mean biomass density, and then calculating a grand mean of those 1000 estimates. We used a bootstrapping method to calculate 95% confidence limits around each of the means. Estimates of bootstrapped mean biomass density of each glider was similar to that using the full data set (Table 5). For example, using the full echo energy data set the mean biomass of krill at 120 kHz the biomass density was within 3 percent of the gridded means for both AMLR03 and AMLR04. In the future, we can refine the grid by estimating the spatial autocorrelation scales, using historical environmental and acoustic data collected during previous ship surveys. This may allow us to refine the grid size to better reflect the inherent scales of patchiness in this area.

Total biomass estimates

The various estimates of krill densities and biomasses in the Bransfield Strait survey area are presented in Tables 3, 4, and 5. Calculated total densities estimates for the Bransfield Strait survey area, ranged from 38.22 to 48.37 g m-2 at 70 kHz and 120 kHz using the full echo energy integration approach, from 33.91 to 41.89 g m-2 at 70 kHz and 120 kHz using the schools approach for both gliders.  The difference between the two approaches is about 13-16% and suggests that most of the acoustic returns at these frequencies is found in schools.

Bootstrapped estimates of densities of krill for all glider profiles were very similar to . . .(Table X and Y).  Estimated densities of krill for the full area ranged from 35.67 to 47.17 g m-2. School densities were lower but similar, ranging from 32.81 to 41.28 g m-2.

# Discussion

We have previously demonstrated that underwater gliders are capable of producing defensible estimates of the biomass of Antarctic krill in Antarctic waters (Reiss et al. 2021). We have also shown using simulation that the sawtooth pattern of glider movement through the water column is robust, and provides similar estimates to ships (Kinzey et al. 2022). Here we have shown that the statistical  properties between two gliders flying the same survey pattern is also robust, and this should provide managers with confidence that data from multi-glider deployments can be easily combined. Finally, we have also demonstrated the use of a new compact single beam wide-band echosounder (Nortek Signature 100 compact echosounder) derived from a similar instrument used on moorings by the U.S. AMLR Program (Cutter et al. 2022).

In a previous version of this ppaer (EMM2024_53) differences in the biomass density estimates between the gliders was attributed to faulty calibrations. The original estimates differed by a factor of 2, and were more than twice te estimates from historical data for the region.

If acoustics are not calibrated or incorrectly calibrated, estimates of biomass can be grossly off. Tesler (1989) noted that calibration error can account for 12 -26 percent of systemic error thus making it an important factor in acoustic analysis.

School analysis has been discussed in during WG-ASAM meetings (WG-ASAM24) as a possible way for scientific and fishing vessels to identify krill, but CCAMLR must ensure that only calibrated echosounders are used in biomass estimation.

Krill school parameters were slightly different than those in Guihen et al. (2014). They tested a number of different parameters as well as comparing EK60 ship schools parameters and a Seaglider equipped with an ES853 single beam echosounder. They noted that glider derived data were coherent with ship-borne measurements using school analysis. Krill schools are being identified deeper than 250m that are most likely not krill. We are still testing the optimal parameters for use with the long ping duration (six seconds) as well as 200m depth range of the Signature 100 compact echosounder. Further investigation of school parameters will be done in the future.

Due to the nature of the glider producing profiles of acoustic backscatter instead of constant data like a ship, we did not separate day and night profiles. To be conservative with possible bubble attenuation we removed the top 10 m of the surface data. This is similar to ship data. In reality, we could just remove the top 5 m of surface data since the likelihood of bubbles accumulating on the face of the transducer is minimal. Gliders may exhibit less bias in sampling krill due to diel vertical migration (Demer and Hewitt 1995) since gliders can begin sampling the water column at ~3 meters. Ships using 120 kHz may not have a good signal to noise ratio past 500 m (De Robertis and Higginbottom 2007) while the glider can use the 120 kHz down to 1000 m making it a valuable tool to sample possible deep euphausiids. Ships are also inherently noisy while a glider is quiet, allow for cleaner acoustic data collection since it is buoyancy driven.

# Conclusions

This is the fifth deployment of echosounder equipped gliders during austral summer around the Antarctic Peninsula, and further demonstrates that these tools are important to continuing acoustic time series that were started in the 1980s.

In addition to providing estimates of biomass density that can be directly used  in the revised krill management plan, the data collected by these instruments have other important implications. For example, we have shown that there are few krill schools below 250m depth, and in general, because the gliders dive to 1000m they reach most of the bottom in the Bransfield strait except in the deep basins away from the fishing areas. Thus, CCAMLR can be more confident that the biomass estimates based on the integration of the upper 250m is sampling a very large fraction of the krill in the water column.

We continue to explore different approaches to the calculation of biomass density and have proposed a modification of the geospatial structuring of sampling, and demonstrated that with even fewer samples in the region (gridded into appropriate cells), the biomass density is still within about 10 to 15% of the estimates of the estimates calculated using the full echo energy data.

In the future, deploying acoustically equipped gliders later in the season may be useful in understanding the changes in vertical distribution in areas away from fishing grounds, as gliders have a long loitering time (months), and can collect data before fishing starts, and until fishing in a region ends. We are working to telemeter acoustic data back to shore during deployments, and this future development will provide us the opportunity conduct adaptive sampling in ecologically important areas.

Moorings

Moorings are another tool that can be used to measure the krill biomass and flux over a long time scale. We have deployed moorings previously ( ref) and deployed one off of Cape Shirreff Dec 2024. This year we plan to deploy nine moorings around the South Shetland Islands (Figure X). These moorings are at 300m water depth and are fitted with upward looking Nortek Signature 100 ADCP-e and SBE37 CTDs with oxygen sensors. The use of more moorings can be expanded to cover more area. We look forward to working with others who are interested in this as well.

References

Coetzee J (2000). Use of a shoal analysis and patch estimation system (SHAPES) to characterise sardine schools. Aquatic Living Resources, 13(1), 1-10.

Cutter Jr, GR, Reiss, CS, Nylund, S, and Watters, GM (2022). Antarctic krill biomass and flux measured using wideband echosounders and acoustic Doppler current profilers on submerged moorings. Frontiers in Marine Science, 9, 784469.

De Robertis, A, and Higginbottom, I. (2007). A post-processing technique to estimate the signal-to- noise ratio and remove echosounder background noise. ICES Journal of Marine Science, 64(6), 1282- 1291.

Demer, DA, Berger, L, Bernasconi, M, Bethke, E, Boswell, K, Chu, D,and Williamson, N (2015). Calibration of acoustic instruments. ICES Cooperative Research Reports (CRR), 215.

Demer, D. A., & Hewitt, R. P. (1995). Bias in acoustic biomass estimates of Euphausia superba due to diel vertical migration. Deep Sea Research Part I: Oceanographic Research Papers, 42(4), 455-475.

Foote K.G., Knudsen H.P., Vestnes G., Brede R. and Nielsen R.L. (1981). Improved calibration of hydroacoustic equipment with copper spheres. ICES CM1981/B:20, 18 pp

Guihen, D, Fielding, S. Murphy, E J, Heywood, KJ, and Griffiths, G. (2014). An assessment of the use of ocean gliders to undertake acoustic measurements of zooplankton: the distribution and density of Antarctic krill (Euphausia superba) in the Weddell Sea. Limnology and Oceanography:

Methods, 12(6), 373-389. https://doi.org/10.4319/lom.2014.12.373

Hewitt, RP, Demer DA, and Emerey, JH. (2003). An 8-year cycle in krill biomass density inferred from acoustics surveys conducted in the vicinity of South Shetland Islands during the austral summers of 1991-1992 through 2001-2002. Aquatic Living Resources. Vol 16, Issue 3. Pp. 205-213. https://doi.org/10.1016/S0990-7440(03)00019-6

Kinzey D, Cossio AM, Reiss CS and Watters GM (2022). Acoustic sampling of Antarctic krill with simulated underwater buoyancy gliders: Does the sawtooth dive pattern work? Front. Mar.

Sci. 9:1064181. doi: 10.3389/fmars.2022.1064181

Miller, AK, and Trivelpiece, WZ. (2007). Cycles of Euphausia superba recruitment evident in the diet of Pygoscelid penguins and net trawls in the South Shetland Islands, Antarctica. Polar Biology, 30, 1615-1623.

Reid, K, and Brierley, A S. (2001). The use of predator-derived krill length-frequency distributions to calculate krill target strength. CCAMLR Science, 8, 155-163.

Reiss, CS, Cossio, AM, Loeb, V, and Demer, DA. (2008). Variations in the biomass of Antarctic krill (Euphausia superba) around the South Shetland Islands, 1996–2006. ICES Journal of Marine Science, 65(4), 497-508.

Reiss CS, Cossio A, Santora JA, Dietrich KS and others (2017). Overwinter habitat selection by Antarctic krill under varying sea-ice conditions: implications for top predators and fishery management. Mar Ecol Prog Ser 568:1-16. https://doi.org/10.3354/meps12099

Reiss CS, Cossio AM, Walsh J, Cutter GR and Watters GM (2021). Glider-Based Estimates of Meso-Zooplankton Biomass Density: A Fisheries Case Study on Antarctic Krill (Euphausia superba) Around the Northern Antarctic Peninsula. Front. Mar. Sci. 8:604043. doi: 10.3389/fmars.2021.604043

SC-CAMLR (2010). Report of the fifth meeting of the Subgroup on Acoustic Survey and Analysis Methods. SC-CCAMLR-XXIX/6.

Tesler, W. D. (1989) Bias and precision in acoustic biomass estimation. Proceedings of the Institute of Acoustics, 11(3), 202-211.

WG-ASAM (2024) Report of the Working Group on Acoustic Survey and Analysis Methods (WG- ASAM-2024).

Tables

Table 1. School detection settings used to estimate the density of krill schools during the 2023/24 austral summer glider deployments using a -75 dB threshold.

| Minimum total school height (meters) | 2 |
| --- | --- |
| Minimum candidate length (meters) | 1.5 |
| Minimum candidate height (meters) | 2 |
| Maximum vertical linking distance (meters) | 2 |
| Maximum horizontal linking distance (meters) | 3 |
| Minimum total school length (meters) | 7.5 |

Table 2. Krill length frequencies from Gentoo penguins on King George Island.

| Length (mm) | Frequency |  | Length (mm) | Frequency |
| --- | --- | --- | --- | --- |
| 20 | 0.000 |  | 41 | 0.073 |
| 21 | 0.000 |  | 42 | 0.063 |
| 22 | 0.002 |  | 43 | 0.037 |
| 23 | 0.000 |  | 44 | 0.044 |
| 24 | 0.000 |  | 45 | 0.077 |
| 25 | 0.000 |  | 46 | 0.042 |
| 26 | 0.000 |  | 47 | 0.066 |
| 27 | 0.005 |  | 48 | 0.044 |
| 28 | 0.002 |  | 49 | 0.028 |
| 29 | 0.007 |  | 50 | 0.044 |
| 30 | 0.009 |  | 51 | 0.028 |
| 31 | 0.009 |  | 52 | 0.028 |
| 32 | 0.005 |  | 53 | 0.005 |
| 33 | 0.023 |  | 54 | 0.005 |
| 34 | 0.035 |  | 55 | 0.009 |
| 35 | 0.063 |  | 56 | 0.005 |
| 36 | 0.044 |  | 57 | 0.002 |
| 37 | 0.033 |  | 58 | 0.000 |
| 38 | 0.049 |  | 59 | 0.000 |
| 39 | 0.047 |  | 60 | 0.000 |
| 40 | 0.063 |  |  |  |

Table 3. Antarctic krill density and biomass estimates, standard deviations and the number of profiles in the survey area.

| Glider | 70 kHz density (g/m2) | 70 kHz STD | 70 kHz biomass (tons) | 120 kHz density (g/m2) | 120 kHz STD | 120 kHz biomass (tons) | Survey profiles (N) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AMLR03 | 48.37 | 106.77 | 4.3533e+05 | 38.22 | 85.64 | 3.4398e+05 | 321 |
| AMLR04 | 43.33 | 117.38 | 3.8993e+05 | 41.67 | 130.78 | 3.7505e+05 | 323 |

Table 4. Antarctic krill density estimates and standard deviation of school analysis in the survey area.

| Glider | 70 kHz density (g/m2) | 70 kHz STD | 70 kHz biomass (tons) | 120 kHz density (g/m2) | 120 kHz STD | 120 kHz biomass (tons) | Number of schools |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AMLR03 | 41.89 | 99.28 | 3.7704e+05 | 33.91 | 78.77 | 3.0515e+05 | 635 |
| AMLR04 | 37.62 | 103.22 | 3.3855e+05 | 35.01 | 99.72 | 3.1508e+05 | 515 |

Table 5. Table of bootstrapped densities with lower and upper confidence intervals and the number of binned data samples for the entire area using 0.2° by 0.1° grid cells.

| Glider | 70 kHz density | Lower CI | Upper CI | 120 kHz density | Lower CI | Upper CI | Number of samples |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AMLR03 | 47.17 | 37.47 | 60.64 | 37.41 | 30.14 | 48.62 | 106 |
| AMLR04 | 37.86 | 28.29 | 51.91 | 35.67 | 27.15 | 51.70 | 101 |
| Schools |  |  |  |  |  |  |  |
| AMLR03 | 41.28 | 31.91 | 55.75 | 33.82 | 26.09 | 44.41 | NA |
| AMLR04 | 34.13 | 25.05 | 49.10 | 32.81 | 23.94 | 46.00 | NA |

Figure 1. Map of the path of AMLR03 and AMLR04 Slocum gliders in the Bransfield Strait during the austral summer deployment in 2023/24, and 0.2° by 0.1° grid overlain on the map used for bootstrapping.

Figure 2. Depth distribution of averaged ABC for AMLR03 (blue) and AMLR04 (orange) using the 70 kHz (A) and 120 kHz (B).

Figure 3. Maps of spatial distribution of krill densities of the 120 kHz for AMLR03 (A) and AMLR04 (B).

Figure 4. Empirical comparisons of acoustic estimates of biomass density from two gliders during the 2023/24 austral summer (A) Histograms of 120kHz log10 ABC for each profile; (B) Empirical cumulative distribution of biomass for both AMLR03 (blue) and AMLR04 (orange).

Look at school metrics to see the differences in patches. School size, density, etc. Does it stay within the beam or past it.